Chapter 4:	Network Connectivity Devices

The interfaces and devices that are used to connect computing devices and transmission media are called connectivity hardware or network connectivity devices.

Network connectivity hardware connects individual devices and transmission media are called connectivity hardware or network connectivity devices”.

- Modern

- Repeaters

- Hubs

- Bridges

- Multiplexers

- Switches

- Routers

- Transmission media connectors etc.

Modem (modulator/ demodulator)

Modem converts your computer digital signal to an analog transmission signal to use with telephone lines or microwave transceivers. Modem is necessary because telephone lines and microwave media uses electromagnetic waves, but your computer uses electric pulses. Modems are also useful when the signal from the transceiver is hot powerful enough to travel a required, distance without significant loss of data, modems can be used to amplify signals.

Repeaters

When an electrical signal is sent across a medium, it fades along the distance (known as attenuation) as a result of resistance from the medium itself. Naturally the longer the distance that it travelled, the more the signal fades. Eventually tie signal fades to a, point where the receiving station cannot recognize the original message (Or has trouble doing so).

In short each transmission medium can be used for a certain distance. However you can exceed the physical medium's maximum effective distance by using an amplification, device called as Repeater. A repeater operates at the physical layer of the OSI model and takes a signal from one LAN and sends it to another LAN- reconditioning and retiming it in the process. The reconditioning usually amplifies and boosts the signal's power. If the signal has travelled a distance it is weak, and so on, the amplification can also be done on noise receivers.

It is important to note that since the repeater has no real knowledge of the data it is carrying, no error checking is performed. Therefore any error are passed from one segment to the next without any ability to stop it.

Hubs

In order to connect various cable segments, we need a central point to plug everything together. A hub is-a multiport repeater. It provides point-to-multipoint connections, it is basically a shared device and works at physical layer of the OSI model. It is often located in a wiring closet and is a point of concentration for wiring.

There are three types of hubs namely,

- Passive hub

- Active hub

- Intelligent hub

A passive hub connects cable segment together. No signal regeneration is performed. So each segment is allowed to be extended to only half the maximum effective distance.

An active hub is like a passive hub except it regenerates or amplifies signals.

The main drawback of active hub is that some active hubs amplify cable noise as well as signal.

An intelligent hub, in addition to signal regeneration, also helps in performing network management functions. The SNMP (simple network management protocol) agents must be embedded in a hub to carry out network related functions.

Hubs generally have RJ-45 ports for unshielded twisted-pair (UTP) cabling, and they range in size from 4 to 24 or more ports for connecting stations to the hub, plus one or more uplink ports for connecting the hub to other hubs in a cascaded star topology. Hubs generally have various light-emitting diode (LED) indicator lights to indicate the status of each port, link status, collisions, and so on. Hubs with several different types of LAN connectors such as RJ-45, BNC, and AUI are commonly called combo hubs.

Multiplexers

Multiplexer combines two or more separate signals onto one high-speed transmission media. It is also known as mux, and is often used to allow remote terminals to communicate with front-end processor ports over a single line. It works at OSI physical layer.

Bridges

Bridges connects two separate networks to form a logical one by operating at the data link layer of the OSI model. Bridges rely on MAC addresses for their operation. Unlike repeaters, bridges examine the packet's destination address before forwarding it to other segments. A bridge extends the maximum distance of your network by connecting separate network segments, and selectively pass signals from one medium segment to another.

Bridges isolate the media access mechanisms of the LANs to which they are connected. If a packet has a destination address on the same network segment as the source of the signal, the bridge ignore the signal. If the destination address is different from the source address network segment, the bridge sends the message along in a fashion similar to what a repeater would. Although they are effective for a small number of LANs, bridges lose many of their benefits as the number of LANs grows. Bridges only operates at the data link layer, and the best source routing information is a component of the network layer.

Bridges offer following advantages over hubs:

- Divide a large network segment into smaller segments and hence reduce data traffic and improves network performance.

- Filter local data traffic by not allowing them to cross other network segments hence reducing overall network traffic.

- Provide exclusive bandwidth (10 Mbps) to each node connected to a port on a bridge as opposed to shared bandwidth provided by hubs.

- Can be used to connect network segments of dissimilar media.

Switches

As a response growing network demands, devices known as switches were introduced to the market in 1991. A switch is a big brother of bridge. The switch is nothing but a large multiport bridge. The switch operates at layer 2 of the OSI model just like a bridge that MAC addresses to determine where to forward the packet. The main differences between switches and bridges, are the strength and speed offered by the switches. Bridges can have maximum 16 ports while switches can offer hundreds of ports, each port offering exclusive bandwidth of 10 or 100 Mbps.

Switches can perform following functions.

- Address learning.

- Filtering and forwarding

- Loop Avoidance

Difference between Hub and Switch

|  | Hub | Switch |
| --- | --- | --- |
| 1. | It functions in a physical layer. | It functions in the data link layer. |
| 2. | Switch allows packet switching. | There is a separate collision domain in the switch. |
| 3. | Hub follows broadcast transmission. | Switch follows three i.e., multicast, unicast, and broadcast type transmission. |
| 4. | In Hub, half duplex transmission technique is utilized. | In switch, full duplex transmission technique is utilized. |
| 5. | Hub does not allow packet filtering. | Switch allows packet switching. |
| 6. | There can be 4 ports in Hub. | 24 to 28 ports contained by a Switch. |

Routers

Routers are the most complicated of the three devices so far, operating at the network layer of the OSI model. While bridges are limited to examine data packets MAC addresses, routers go beyond this and-can examine the network address-which has routing information encoded in it. Routers can use this information to make intelligent decisions about routes and paths.

In the simplest form routers like bridges-can be used to connect network segments. Whereas bridges only know to forward what they don't recognized, routers are aware of multiple paths that lead to a destination address and know which path is best.

Each network segment is assigned a specific address and is then referred to as a sub network or subnet. Each node on the network is then assigned an address.

Every data packet sent contains the destination network address and node address.

The optimum path can then be determined by looking at infernal routing table. One of the biggest differences between bridges and routers is the ability to identify where data is going, the router must initialize and maintain the routing table and determine the next hop in the packet’s journey, a router is expected to be able to identify the address and only send packets for which it has a network address. If a machine address isn't found in the routing table, the packet is discarded.

To get at the network layer and find the information it needs, the router must first strip off the Data Link Layer. After it finds the information, it repackages the data packets. A key advantage of routers comes into play during this operation: Since the data is unpacked and repacked, there wan opportunity transform the data to the data frame needed for a particular architecture.

Routers are normally responsible for performing the following functions:

• Route selection -

A router is maintaining the information in its routing table about how to reach remote networks. It will then make routing decisions based on that information

• Logical addressing-

A device that operates at layer 3 requires some form of logical addressing. These addresses will be used to determine route selection.

• Segmentation

Routers can be provide by a powerful method of segmenting your networks to allow optimum utilization of available bandwidth.

Advantages of using Routers

• No broadcasts

Because router operates at Layer 3 of the OSI mode), no Layer 2 broadcasts wiII be forwarded through a router.

• Manageability

Routers have a better knowledge of the network topology than bridges and switches do and have the ability to support more protocols than bridges and switches

• Increased bandwidth

By segmenting your networks with routers, your nodes/ hosts will have more access bandwidth

• Packet fragmentation and reassembling

Routers provide, packet fragmentation / reassembly functions, as well as better

Security.

Gateways

Gateways are often lumped into discussion about bridging and routing, when in fact the-service they perform is similar but different by one major factor: with a gateway, data is translated between two different data formats or network architectures.

Gateways perform much higher-level translations than any other component and thus work at the Application layer of the OSI module. When packets arrive at a gateway, all the information is stripped off the data until it reaches the layer where it can translate the information-using the format needed for the destination